NETWORKING MULTIPLE-CHOICE QUESTIONS

1. Which hardware choice best supports traffic separation and efficient forwarding in a departmental network?

☐ Repeater

☐ Hub

☐ Switch

☐ Modem

2. A subnet is full, and new hosts must be added without redesigning the whole network. What should the administrator do?

☐ Remove the switch

☐ Reuse duplicate IPs

☐ Disable routing

☐ Replan the addressing strategy

3. A company do not want to separate finance, HR, and student lab traffic, but still allow all three to use one building's wiring. Which solution is best?

☐ A. One flat LAN

☐ B. VLAN segmentation

☐ C. Hub cascade

☐ D. Shared public subnet only

4. A network administrator wants to limit the size of broadcast domains while keeping routing manageable. What is the best strategy?

☐ Design a Flat network

☐ Subnetting and hierarchical design

☐ Deploy Home routers

☐ One switch per host

5. A company is expanding its network. VLANs 10 (ADMIN), 20 (SALES), and 30 (IT) have already been created. The administrator needs to configure Fa0/18 as an access port for the IT department and connect it to a user workstation. Which configuration is the best choice?

☐ A.
Switch(config)# interface fa0/18
Switch(config-if)# switchport mode trunk
Switch(config-if)# switchport trunk allowed vlan 30

☐ B.
Switch(config)# interface fa0/18
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 30

☐ C.
Switch(config)# vlan 30
Switch(config-vlan)# interface fa0/18

☐ D.
Switch(config)# interface vlan 30

6. A network architect places highspeed backbone equipment in the top layer of a hierarchical design. Why?

☐ To keep critical traffic fast and resilient

☐ To increase collisions

☐ To reduce user authentication

☐ To eliminate addressing

7. A technician notices that PCs connected to Fa0/10 are placed in the wrong VLAN. The intended VLAN is VLAN 40. Which command should be used to correct the assignment?

☐ A.
Switch(config)# interface fa0/10
Switch(config-if)# switchport access vlan 40

☐ B.
Switch(config)# interface fa0/10
Switch(config-if)# switchport mode trunk

☐ C.
Switch(config)# vlan 40
Switch(config-vlan)# interface fa0/10

8. A technician sees that a whole department goes offline when one switch port group fails. What design issue is most likely present?

☐ Too much segmentation

☐ Poor redundancy at the access layer

☐ Too many VLANs set in the switch is not used

☐ Excessive fiber use

9. During a network upgrade, the admin plans a structure where the access layer connects end devices, the distribution layer enforces policy, and the core layer carries fast backbone traffic. What design is being used?

☐ Bus design

☐ Peertopeer design

☐ Hierarchical network design

☐ Ring design

10. A hospital needs a network that keeps patient records secure, survives device failure, and can expand to a new wing later. Which design choice best addresses all three?

☐ One hub-based LAN

☐ Direct PC to PC connections

☐ Hierarchical design

☐ Flat topology with one subnet

11. A university wants guest WiFi separated from internal systems on the same infrastructure. Which design is best?

☐ Limit IP addresses

☐ VLAN segmentation

☐ One broadcast domain for all users

☐ Hub based topology

12. A school wants to isolate students from faculty but still use the same physical infrastructure. Which design best supports this?

☐ Hub based network

☐ Peer to peer only

☐ Single broadcast domain

☐ VLAN based segmentation

13. Two departments share one switch but should not see each other's broadcasts. What is the best solution?

☐ Hub broadcasting

☐ Set one secure router

☐ Separate VLANs

☐ Disable un-used IP addresses

14. A company wants to avoid a single point of failure in its backbone. Which choice best supports reliability?

☐ A single long cable run

☐ Redundant core paths and devices

☐ One hub for all connections

☐ One core switch with no backup

15. A network has many collisions, poor performance, and no logical segmentation. What is the most effective redesign?

☐ Merge departments that use and process same data.

☐ Replace cables with high transmission rate

☐ Replace hubs with switches and add VLANs

☐ Add more hosts to the same segment

16. A designer wants a network that can continue operating when one path fails. Which principle is being prioritized?

☐ Portability

☐ Reliability

☐ Compression

☐ Centralization

17. A technician has created VLAN 30 named ACCOUNTING. PC-A is connected to FastEthernet0/5 and must belong to VLAN 30. Which configuration should be applied?

☐ A.
Switch(config)# interface fa0/5
Switch(config-if)# switchport mode trunk
Switch(config-if)# switchport trunk allowed vlan 30

☐ B.
Switch(config)# interface fa0/5
Switch(config-if)# switchport access vlan 30

☐ C.
Switch(config)# interface fa0/5
Switch(config-if)# vlan 30

☐ D.
Switch(config)# vlan 30
Switch(config-vlan)# interface fa0/5

Answer Key

1. Switch

2. Replan the addressing strategy

3. A. One flat LAN

4. Subnetting and hierarchical design

5. B

6. To keep critical traffic fast and resilient

7. A

8. Poor redundancy at the access layer

9. Hierarchical network design

10. Hierarchical design

11. VLAN segmentation

12. VLAN based segmentation

13. Separate VLANs

14. Redundant core paths and devices

15. Replace hubs with switches and add VLANs

16. Reliability

17. B